Basic Airplane Proportions…

The wing length should be about 5-6 times the width. The width of the wing from leading edge to trailing edge is called the CHORD.

The length of the body of the airplane (called the FUSELAGE) should be about 70-75% of the wing length.

The distance from the leading edge of the wing to the propeller should be about 15% of the wingspan. (wing length)

The distance from the leading edge of the wing to the horizontal stabilizer should be about 3 times the chord of the wing.

The horizontal stab should be about 25% of the wing area.

The vertical stabilizer should be about 10% of the wing area.

The plane should balance at about 25-33% of the wing chord.