'		Close Encounters of the Curiously Minty Kind

'		Eat your heart out Spielberg!

'

'		Andy Gadget - 2009

'

'		12/9	Count 'dormant' loop and change random seed

'			Eliminate zero values from random LED flash

‘			253 of 256 bytes used.

#picaxe 08m					'Tells the editor the target chip

#freq m8					'Ups the internal oscillator to 8MHz

data 0,(23,25,21,5,144,255) 			' main tune

data (161,33,207,97,97,97,97,255)		' response 1

data (74,80,72,104,195,255)			' tune 2

data (161,97,97,97,97,97,98,15,33,255)	' response 2

data (87,89,21,229,128,255)			' ship joins in

data (23,25,21,165,128,255)			' final phrase

'Behaviours:

' Slow pulse on turn-on

' Wait until magnet positioned

' Run main theme

' Run random data until.magnet removed

' Takeoff!

'	w6	b13	rnd_h

'	w6	b12	rnd_l

'	w5	b11	cnt

'	w5	b10	tmp1

'	w4	b9	tmp2

'	w4	b8	tmp3

symbol x = output0		'x is LED bank 1

symbol y = output1		'y is LED bank 2

symbol z = output4		'z is LED bank 3

symbol aud = output2		'Audio output

symbol sw = pin3	 	'Reed switch input

symbol rnd   = w6		'16 bit random number storage

symbol rnd_h = b13		'High byte of random number

symbol rnd_l = b12		'Low byte of randon number

symbol cnt = b11		'Loop counter

symbol tmp1= b10		'Scratch variable

symbol tmp2 = b9		'Scratch variable

symbol tmp3 = b8		'Scratch variable

symbol oldval = b5		'Var to test for repeat pattern

symbol newval = b4		'Var to test for repeat pattern

'Audio always on output2

low x,y,z

main:

if sw = 1 then

gosub Dormant			'Slow pulse

else

gosub PlayIt 			'Tune sequence

gosub RandomData

goto Takeoff

endif

goto main

PlayIt:

wait 3

for cnt=0 to 41			'Loads data sequentially from beginning of EEPROM

read cnt,tmp3

if tmp3 = 255 then 		'Value 255 indicates end of current line

low x,y,z			'Turn LEDs off

wait 3			'Wait before playing next line

else

gosub RndLed		'Set a random pattern on LEDs

tune 0,5,(tmp3)	'Play the note loaded from memory

pause 50		'50mS pause

endif

next cnt

wait 3

return

RndLed:

do

random w6			'Use the RANDOM command to gen. 16bit number

newval=rnd_h & %00010011	'Mask high byte to correspond to LED outputs

loop until newval <> oldval AND newval <>0

'Loop if value is the same as last time or zero

pins = newval

oldval=newval

return

Dormant:

servo x,0				'SERVO comand is normally used for driving

servo y,0				'RC servos, but I'm using it as PWM source

servo z,0				'to pulse LED brightness

for tmp1 = 1 to 254		'Loop up to maximum servo position

servopos x,tmp1

servopos y,tmp1

servopos z,tmp1

pause 10

next tmp1

tune 0,1,($5b)			'High beep

for tmp1 = 254 to 1 step -1 	'Loop down to minimum servo position

servopos x,tmp1

servopos y,tmp1

servopos z,tmp1

next tmp1

tune 0,1,($60)			'Low beep

low x,y,z			'Redefine the LED pins as normal outputs

inc w6				'Increment variable to count loops before stimulus applied.

return

RandomData:

do

tune 0,1,(rnd_l)		'Play a note using low byte of random number

gosub RndLed			'Call the random LED flash routine

loop until sw=1			'until the stimulus magnet is removed

low x,y,z				'Turn off all LEDs

wait 5					'and wait for 5 seconds

return

TakeOff:

for b3 = 1 to 127			'This uses nested loops to give alternating tones

b1=b1+1				'and uses the fact that the SOUND command starts off

b2=b1+20			'with tones and then changes to modulated noise

for cnt=b1 to b2		'at values above 127.

high x,y,z

sound aud,(cnt,1)

next cnt

for cnt=b2 to b1 step -1

sound aud,(cnt,1)

low x,y,z

next cnt

next b3

low x,y,z				'Turn everything off

wait 60						'and go to sleep for a minute.