'

'		Tic Tac DrumBox - (c) Andy Stansfield 19/7/2009 (Andy Gadget)

'		Released under Attribution - Non commercial - No derivs license

'

'		Touch anywhere on the top track to turn on.

'		It will then compose and play beat rhythms (1 to 5 beats) at various tempos.

'		It composes two bars full and chooses between these randomly.

'		It starts a new rhythm approximately every 30 seconds.

'

'		Turn off by touching the top track again.

'

'		Note : This powers up in 'sleep' mode and wakes every 4 seconds to see if

'		anything is happening.  It may not be awake when the PC tries to communicate.

'		Simplest way around is to turn it on first -

#picaxe 08m

data (0,0,0,0,0,0,0,0,0,0)		'reserve space for beat data (2 bars worth)

symbol rnd   = w6			'16 bit random number storage

symbol rnd_h = b13		'High byte of random number

symbol rnd_l = b12		'Low byte of randon number

symbol led	 = 1

symbol cnt   = b11		'Loop counter

symbol beats = b10		'Beats in  measure

symbol tempo = b9			'Tempo of song

symbol stime = b8			'Number of measures in song

symbol instr = b7			'Instrument number

symbol rest	 = b6			'Pause to set tempo

symbol adc   = b4			'ADC reading - turns on and sets random seed.

symbol tmp3	 = b2			'Temp variable

symbol tmp2	 = b1			'Temp variable

symbol tmp1	 = b0			'Temp variable

PwrSave:

do

sleep 4

readadc 4, adc				'Read value from adc

loop until adc>128			'Switch on if top track touched

rnd_l = adc				'ADC value sets seed for RANDOM command

rnd_h = rnd_l				'Double up to make word from bytes

random rnd				'Gen.first 16 bit random number from seed

tune 0,4,(84,89)				'Beep to acknowledge turn-on

pause 200

do						'This is the start of the main loop

'Set up the rhythm strings

beats = rnd_l AND %00000011 + 2   	'2 to 5 beats

tempo = rnd_h AND %00001111 + 1	'1 to 8

stime = 512 / beats / tempo	'Time for tune will vary with beats and tempo

rest = tempo * 50			'Less rest between beats = faster tempo

tmp1 = beats * 2 - 1

for cnt = 0 to tmp1

random w6

instr = rnd_l AND %00000111	'Random selection of one of 8 notes,

write cnt,instr			'Store it in data area at beginning of memory

next cnt

for cnt = 1 to beats			'LED flashes to show beats of rhythm before play

high led

pause 5

low led

pause 200

next cnt

' Play the rhythm

for tmp3 = 1 to stime			'Stime is rhythm run count

random w6

tmp1 = rnd_h AND %00000001 * beats 'Pointer to phrase is 0 or beats value

tmp2 = tmp1 + beats			'so selects phrase to play

for  cnt = tmp1 to tmp2

read cnt,instr			'Read the instrument to play from data area

on instr goto snare,cowbell,trill,shake,scrape,lobel,midbel,hibel

cont:

pause rest

'Using goto instead of gosub saves 14 bytes!

next cnt

readadc 4, adc					'Read value from adc

if adc > 128 then pwrsave		'Shut down if top track touched

next tmp3

loop

snare:

sound 2,(254,3,240,3,230,8)			'These are the various percussion sounds.

goto cont

cowbell:

sound 2,(120,1,110,2,0,11)

goto cont

trill:

sound 2,(122,1,10,2,122,1,10,2,0,8)

goto cont

shake:

sound 2,(200,1,255,10,0,3)

goto cont

scrape:

sound 2,(250,3,251,3,252,3,254,5)

goto cont

lobel:

sound 2,(120,4,0,10)

goto cont

midbel:

sound 2,(122,4,0,10)

goto cont

hibel:

sound 2,(124,4,0,10)

goto cont