1/ Both vehicles:

a. engines off

b. handbrakes applied

c. gear in neutral

d. open hoods

e. locate battery terminals

3/ start the engine of the good vehicle

2/ Connect cables:

a. red cable to bad battery [+] first, then good battery [+]

b. black cable to good battery [-] first, then bad battery [-]

4/ Attempt to start the engine of the bad vehicle

- turn over for not more than 5 seconds, maximum 3 times

- rest for 1 minute and attempt again if it fails to start (5 tries)

5/ Carefully disconnect cables (one at a time),

- close hoods, repack cables and instruction sheet,

- thank the person who helped you

6/ Drive for 20 km / 12 miles

+        -

-          +

b.

a.

red

black