Gyro Horizon (Attitude Direction Indicator)

Abstract:

Gyro Horizon, also called ADI (Attitude Direction Indicator) or Artificial Horizon is an important aviation instrument in the aircraft cockpit. It gives the pilot an indication of the aircraft attitude (Pitch and Roll) during flight.

This is the actual ADI of the commercial jet Embraer 170 which was my inspiration

Here are some digitally generated simulations to illustrate how the ADI functions and looks like in the aircraft cockpit in front of the pilot.

Leveled flight

Roll Right

Roll Left

Pitch Up

Pitch Down

Unlike fighter jets, commercial jets are unable to fly upside down, but this is a computer generated picture to illustrate how the ADI would respond to such a situation.

Renesas RDK board comes with many types of sensors. I used the motion sensor (3D accelerometer) to sense the Roll and Pitch attitude. The 3D accelerometer acts as a tilt sensor, the Microcontroller analyses the values and then draws the output on the board LCD.

The contrast of the LCD can be adjusted by the included potentiometer.

Theory of operation:

A 3d accelerometer is one member of MEMS family which senses acceleration in 3 dimensions. The entry makes use of the included accelerometer by sensing the tilt in Roll channel and sensing the Pitch channel due to the effect of variation of gravity component on the accelerometer.

In this application, the accelerometer is used as a tilt sensor (as a Gyroscope).

The Microcontroller then uses the Glyph library to draw the ADI shape on the LCD and indicates the Roll attitude.

Version 2

This version differs from Version 1 in one important feature. The first version of the software only indicated the Roll attitude ( Roll Right and Roll Left ) . But this new version senses and indicates the Roll and also the Pitch attitude of the aircraft ( Pitch Up [ Nose Up ] and Pitch Down [ Nose Down ] ).

Note:

This application is written for the Renesas RDK contest and is not intended to be used as an aircraft navigation aid.